Pen Medication Injections

This resource document is meant to be a tool for healthcare providers to post in their building or include in a location that is easily accessible for those administering these medications.

|  | Medication | Discard Date After 1st Administration | Prime/Air Dose Units* | Time Frame to Hold Down After Administered | Additional Recommendations |
| --- | --- | --- | --- | --- | --- |
| Insulin | Humalog | 28 days | 2 units, no more than 4 times | Count to 5 |  |
|  | Tresiba | 56 days | 2 units, no more than 6 times | Count to 6 |  |
|  | Lantus | 28 days | 2 units, no more than 3 times | Count to 10 |  |
|  | Levemir | 42 days | 2 units ** | Count to 6 |  |
|  | Humulin N Kwikpen | 14 days | 2 units, no more than 4 times | Count to 5 | Before putting the needle on you must roll the pen 10 times and then invert the pen 10 times. |
|  | Novolog | 28 days | 2 units, no more than 3 times | Count to 10 |  |
|  | Toujeo Max Solostar | 56 days | 4 units to prime ** | Count to 5 | Remove a new pen from the refrigerator at least 1 hour before use. |
|  | Toujeo Solostar | 56 days | 3 units to prime ** | Count to 5 | Remove a new pen from the refrigerator at least 1 hour before use. |
| Human GLP-1 Receptor Agonist | Ozempic | 56 days | Use the flowcheck symbol on the pen | Count to 6 |  |
|  | Trulicity | Pen can be stored up to 14 days at room temperatures | NA | Until 2nd click is heard | Single dose pen, turn the pen to the unlock symbol, press the button on end while holding the clear base firm. |
|  | Mounjaro | Can be stored unrefrigerated up to 21 days. | NA | Until 2nd click is heard | Single dose pen, turn the pen to the unlock symbol, press the button on end while holding the clear base firm. |